EE211 2025 Fall

Homework - Problem Set #4

Due: Nov 17th, 11:59 pm

Please scan your homework and upload it on the KLMS website.

Problem 1. (10 points)

Derive the Einstein Relation. (Equation 5.47 in the textbook )

Sol)

If we consider the nonuniformly doped semiconductor and assume there are no electrical connections so that the semiconductor is in thermal equilibrium, then the individual electron and hole currents must be zero.

Jn=0=enμnEx+eDndndx … (1.1)

If we assume quasi-neutrality where n can be expressed as Nd (x) , we can rewrite Equation (1.1) as

Jn=0=eμnNdxEx+eDndNd(x)dx … (1.2)

By substituting Ex from Equation (1.1) into Equation (1.2), we obtain

0=-eμnNdxkTe1NdxdNdxdx+eDndNd(x)dx … (1.3)

Equation (1.3) is valid for the condition

Dnμn=kTe … (1.4)

The hole current must also be zero in the semiconductor. From this condition, we can show that

Dpμp=kTe … (1.5)

Einstein relationship: Dnμn=Dpμp=kTe

Problem 2. (10 points)

Answer the following questions as concisely as possible.

(a) Is linear relationship between the drift velocity and the electric field always valid? (2pts)

(b) Name two types of scattering mechanisms that impact carrier mobility and provide a brief explanation of them. (2pts)

(c) What is the relationship between the hole diffusion current density and the gradient of the carrier concentration in a semiconductor? (3pts)

(d) Write the equation for the total current density in a semiconductor device for the one-dimensional case and provide an explanation of its components. (3pts)

Sol)

(a) The linear relationship between Jdrf and E is not always valid because at high electric fields, carrier velocities can saturate or even decrease due to collisions with ionized impurity atoms and thermally vibrating lattice atoms.

(b) Carrier mobility is affected by various scattering mechanisms that cause carriers to deviate from their average velocity.

1. lattice (phonon) scattering: the interaction between carriers and lattice vibrations (phonons).

2. Ionized impurity scattering: the Coulomb interaction between carriers and ionized dopant atoms.

(c) Jpx|dif = -eDp(dp/dx),

The negative sign indicates that the diffusion current flows from high concentration to low concentration, opposite to the direction of the concentration gradient.

(d) J = enunEx + epupEx + eDn(dn/dx) - eDp(dp/dx),

The total current density in a semiconductor device is the sum of the drift and diffusion current densities, where n and p indicate the electron and the hole respectively.

Problem 3. Chenming Hu (10 points)

(a) For an electron mobility of 500 cm2/V∙s, calculate the time between collisions. (Take mn=m0 in these calculations)

(b) For an electric field 100 V/cm, calculate the distance an electron travels by drift between collisions.

Sol)

(a) The mean free time between collisions is

μn=qτmnmn

τmn=μnmnq=2.84×10-13 sec

(b) Drift velocity

vd=μnε=50000 cm/sec

The distance traveled by drift between collisions is

d=vdτmn=0.14nm

Problem 4. Textbook problem 5.6 (10 points)

Consider a homogeneous gallium arsenide semiconductor at T = 300 K with Nd = 1016 cm-3 and Na = 0. (Suppose ni = 1.8 x 106 cm-3 , μn = 7500 cm2 /V-s)

(a) Calculate the thermal-equilibrium values of electron and hole concentrations. (2.5pts)

(b) For an applied E-field of 10 V/cm, calculate the drift current density. (2.5pts)

(c) Repeat parts (a) and (b) if Nd = 0 and Na = 1016 cm-3. (5pts)

(Suppose ni = 1.8 x 106 cm-3 , μp = 310 cm2 /V-s)

Sol)

(a) no=Nd=1016 cm-3 and po=ni2no=1.8×10621016=3.24×10-4cm-3

(b) J=eμnnoE

For GaAs doped at Nd=1016 cm-3 μn=7500cm2 / V_s

Then J=1.6×10-197500101610=120 A/cm2

(c) po=Na=1016 cm-3 and no=ni2po=1.8×10621016=3.24×10-4cm-3

For GaAs doped at Na=1016 cm-3 μp=310cm2 / V_s

J=1.6×10-19310101610=4.96 A/cm2

Problem 5. (10 points)

The drift velocity graph of Si is plotted as below. (μn of Si: 1350 cm2/V-s)

(a) A Si bar with a length of 1 μm and a cross-sectional area of 100 μm2 is uniformly doped with phosphorous at 1017 cm-3. Find the current at 300K when 10V is applied.

(b) Find the time for an electron to drift 1 μm in pure Si for an electric field of

100 V/cm. Also, find a limit (minimum value) of this electron drift time.

Sol)

(a) Applied electric field: E=10V10-4 cm=105 V/cm.

Velocity saturation occurs at the given electric field.

I=eAnvsat=1.6×10-19C×10-6 cm2 × 1017 cm-3 ×107cms=0.16 A

(b) As the 100V/cm is low electric field where velocity is linear to the electric field,

vd=μnE=1350×100=1.35×105 cm/s .

t=Lvd=10-4cm1.35×105 cm/s  =7.4×10-10 s=0.74 ns.

t≥tlimit=Lvdsat=10-4cm107cms =10 ps.

Problem 6. Textbook problem 5.18 (10points)

An n-type silicon resistor has a length L=150 μm, W=7.5 μm, and thickness T=1 μm. A voltage of 2 V is applied across the length of the resistor. The donor impurity concentration varies linearly through the thickness of the resistor with Nd=2×1016cm-3 at the top surface and Nd=2×1015cm-3 at the bottom surface. Assume an average carrier mobility of μn=750cm2/V∙s

(a) What is the electric filed in the resistor?

(b) Determine the average conductivity of the silicon.

(c) Calculate the current in the resistor.

(d) Determine the current density near the top surface and the current density near the bottom surface.

Sol)

(a) E=VL=2150×10-4=133.3 V/cm

(b) σx=eμnNdx

σavg=eμn∙1T0T2×10161-x1.111Tdx

=eμn2×1016Tx-x221.111T0T

=eμn2×1016TT-T221.111T

=eμn2×10160.55

=1.32 Ω∙cm-1

(c) I=σavgAL∙V=1.32×10-5 A

(d) Top surface:

σ=1.6×10-197502×1016=2.4 Ω∙cm-1

J=σE=2.4133.3=320 A/cm2

Bottom surface:

σ=1.6×10-197502×1015=0.24 Ω∙cm-1

J=σE=0.24133.3=32 A/cm2

Problem 7. Textbook Problem 5.31 (10 points)

The electron diffusion current density in a semiconductor is a constant and is given by Jn = - 2 A/cm2. The electron concentration at x = 0 is n(0) = 1015 cm-3.

(a) Calculate the electron concentration at x = 20 um if the material is silicon with Dn = 30 cm2/s. (6pts)

(b) Repeat part (a) if the material is GaAs with Dn = 230 cm2/s. (4pts)

Sol)

(a) Jn = eDn(dn/dx) = eDn(∆n/∆x)

-2=1.6×10-19301015-nx10-20×10-4

4×10-3=4.8×10-3-4.8×10-18nx1

which yields, nx1=1.67×1014cm-3

(b) -2=1.6×10-192301015-nx10-20×10-4

4×10-3=3.68×10-2-3.68×10-17nx1

which yields, nx1=8.91×1014cm-3

Problem 8. Textbook Problem 5.40 (10 points)

Consider an n-type semiconductor at T = 300 K in thermal equilibrium (no current). Assume that the donor concentration varies as Nd(x) = Nd0*e-x/L over the range 0 ≤ x ≤ L where Nd0 = 1016 cm-3 and L = 10 um.

(a) Determine the electric field as a function of x for range 0 ≤ x ≤ L. (5pts)

(b) Calculate the potential difference between x = 0 and x = L (with the potential at x = 0 being positive with respect to that at x = L). (5pts)

Sol)

(a) Ex=-kTe∙1Ndx∙dNdxdx

= -0.0259Ndoe-xL∙ddxNdoe-xL

= -(0.0259)Ndoe-x/L∙-1LNdoe-xL=0.0259L=25.9 V/cm

(b) ϕ=-0LExdx=-25.9L-0= -0.0259 V= -25.9 mV

Problem 9. Textbook problem 5.43 (10points)

In GaAs, the donor impurity concentration varies as Ndoexp⁡(-xL) for 0≤x≤L, where L=0.1 μm and Ndo=5×1016cm-3. Assume μn=6000 cm2/V∙s and T=300 K.

(a) Derive the expression for the electron diffusion current density versus distance over the given range of x.

(b) Determine the induced electric field that generates a drift current density that compensates the diffusion current density.

Sol)

(a) We have

Jdiff=eDndndx=eDndNdxdx=eDn-LNd0exp-xL

Dn=μnkTe=60000.0259=155.4 cm2/s

Then,

Jdiff=-1.6×10-19155.45×10160.1×10-4exp-xL=-1.243×105exp-xLA/cm2

(b)

0=Jdrif+Jdiff

Now,

Jdrif=eμnnE

=1.6×10-1960005×1016exp-xLE

=48∙exp-xLE

We have

Jdrif=-Jdiff

So, 48∙exp-xLE=1.243×105exp-xL

Which yields

E=2.59×103V/cm

Problem 10. Textbook problem 5.48 (10 points)

A semiconductor Hall device at T = 300 K has the following geometry:

d= 10-3cm, W= 10-2 cm, and L= 10-2 cm.

The following parameters are measured: Ix = 0.50 mA, Vx =15V, and Bz =0.10 tesla.

Determine the

(a) conductivity type (n-type vs p-type with appropriate reason),

(b) majority carrier concentration, and

(c) majority carrier mobility.

(d) Derive the electron concentration and the mobility of a n-type semiconductor in Lorenz frame with VH, L, W, Bz, d and Ix (Hall effect). (2.5pts)

Sol)

(a)    VH<0 :n-type

(b)  n=-IxBzedVH=-0.50×10-30.101.6 ×10-1910-5-5.2×10-3=6.01×1021m-3

(c)  μn=IXLenVXWd=0.5×10-310-31.6 ×10-196.01 ×10-211510-410-5

=346.6 cm2 / V-s

(d) The magnetic field force will be exactly balanced by the induced electric field force.

This may be written as F=qE+v×B=0 which becomes qEy=qvxBz

The induced electric field in the y direction is called the Hall field. The Hall field produces

a voltage across the semiconductor which is called the Hall voltage.

VH=+EHW=vxWBz where+y:positive EH for p-type

For a n-type semiconductor, drift velocity of electrons can be written as

vdx=Jxen=Ix(en)(Wd)

As the Hall voltage for n-type is VH=-vdxWBz=-IxBzned

Thus, the electron concentration is n=-IxBzedVH

For low-field majority carrier mobility, Jx=enμnEx→IxWd=enμnVxL→μn=IxLenVxWd