EE211 Introduction to Physical Electronics

Spring 2023

Midterm Exam

April 17, 2023

Monday 16:00 – 17:30 am

- CLOSED BOOK

- Show all your work! Partial credit may be given.

- No calculator and no cell phone.

- You have 1.5 hours to complete this exam.

- There are total of 6 problems.

| Problem | Score Assigned | Score Earned |
| --- | --- | --- |
| 1 (T, F) | 20 |  |
| 2 | 20 |  |
| 3 | 10 |  |
| 4 | 15 |  |
| 5 | 15 |  |
| 6 | 20 |  |
| Total | 100 |  |

GOOD LUCK!

Problem 1.  Conceptual True/False (20 points (2 points each))

Read following descriptions and answer with writing T or F depending on your decision. Correct the inclined part when your answer is F.

|  |  | T/F | Corrections |
| --- | --- | --- | --- |
| 1 | The conventional unit cell of the face-centered cubic (FCC) lattice is not a primitive cell. The distance between the lattice points is so called the lattice constant. |  |  |
| 2 | The eigenfunctions of a particle in a 1-D infinite potential box are ψn=2LsinnπxL. When the particle is the first excited state (n = 2), the probability finding the particle at x= L/2 equals to 0. |  |  |
| 3 | Effective mass of electron is smaller than that of hole in silicon. This makes the density of state in conduction band larger than that in the valance band. |  |  |
| 4 | Since the momentum of photon is very small as calculated in HW, the direct bandgap semiconductors in general have larger optical absorption and emission coefficients than the indirect bandgap ones. |  |  |
| 5 | Effective mass of electrons at the top of the valance band in silicon has negative value and the absolute value is larger than that of electrons at the bottom of the conduction band. |  |  |
| 6 | The intrinsic carrier density of Si is increased with the increase of temperature. |  |  |
| 7 | In a p-type semiconductor, the most of the holes in the valance band are created with creating electrons in the conduction band. |  |  |
| 8 | Majority carrier concentration of an extrinsic semiconductor decreases in an extrinsic region as the temperature increases. |  |  |
| 9 | As temperature increases in a n-type doped semiconductor, the Fermi level moves toward to conduction band edge. |  |  |
| 10 | The n0p0 product in a semiconductor does not depend on doping density in equilibrium and does depend on semiconductor materials. |  |  |

Problem 2. Atomic packing factor (20 points)

Consider atoms with radii r equal to 1/2 of the distance between nearest neighbors. Calculate the ratio of the volume that is filled by the atoms to the total volume in the (a) simple cubic lattice, (b) body centered cubic lattice, (c) face centered cubic lattice, and (d) diamond lattice.

Problem 3. Density of states (10 points)

The density of states (DOS) of electrons in a 1D wire is known to be

,

where m is the effective mass of electrons. Derive it.

Problem 4. Effective mass and optical transition in silicon (15 points)

The minimum energy in the conduction band of silicon is appeared in the [100] direction in k-space. The energy in this one dimensional direction near the minimum value can be approximated by    Ek(Conduction)= EC+ Ec1(k-ko)2+Ec2(k-ko)3.  The energy near the maximum value at k = 0 in the valance band can be approximated by Ek(Valence)  = EV-Ev1k2+ Ev2k3.

- What is the effective mass for an electron near the bottom of the conduction band? (7 points)

- When electrons on the top of the valance band are excited to the bottom of the conduction band, explain this transition mechanism in terms of energy and momentum conservations with the equation parameters. (8 points)

Problem 5. Calculation of donor electron ionization energy in silicon (15 points)

For a hydrogen atom, the radius (Bohr radius) of the orbiting electron is given as

a0=4πϵ0ℏ2m0e2

and the ionization energy (Rydberg) of an electron is given as:

E1=-m0e42(4πϵℏ)2

Explain how the electron radius and the ionization energy are approximately given for a donor electron in silicon. Explain quantitatively how they are different from a0 and E1 (should be correct up to the order of magnitude).

Hint: E1 = -13.6 eV, a0= 0.53 A, r (silicon) = 11.4, and m*(effective mass of electron in Silicon) = 0.26 m0

Problem 6. Carrier concentration and Fermi level position with respect temperature (20 points)

Schematically draw the logarithm of electron concentration in Si as a function of (a) temperature and (b) inverse temperature. Explain the origins of the three distinct regions and in (b) the slope for the low-temperature range. (c) Schematically draw the shift of Fermi level within the band gap below each figure, (as a function of temperature and inverse temperature).