EE211 Spring 2024

Homework - Problem Set #5

Due: Nov 26th, 11:59 pm

Please scan your homework and upload it on the KLMS website.

Problem 1 (10 points)

Explain qualitatively why a pulse of excess electrons and holes would move together in the presence of an applied electric field.

<Solution>

With charge neutrality, δn = δp at any instant of time and at any point in space. The excess electron concentration is equal to the excess hole concentration. Thus, the excess electron pulse is moving in the same direction as the applied electric field even though the electrons have a negative charge.

Problem 2 (10 points)

GaAs, at T = 300 K, is uniformly doped with acceptor impurity atoms to a concentration of Na = 21016 cm-3. Assume an excess carrier lifetime of 510-7s.

- Determine the electron-hole recombination rate if the excess electron concentration is n = 51014 cm-3.

- Using the results of part (a), what is the lifetime of holes?

<Solution>

po=Na=2 × 1016 cm-3, n0=ni2p0=(1.8 × 106)22 ×1016=1.62 × 10-4 cm-3

(a)  R'=δnτno=5 ×10145 ×10-7=1021 cm-3s-1

(b)  Rp=poτpt=noτnt=noτno

τpt=ponoτno=2 ×10161.62 ×10-4×5 ×10-7=6.17 × 1013s

Problem 3 (10 points)

An n-type silicon sample contains a donor concentration of Nd = 1016 cm-3. The minority carrier hole lifetime is found to be τp0 = 20 μs.

- What is the lifetime of the majority carrier electrons?

- Determine the thermal-equilibrium generation rate for electrons and holes in this material.

- Determine the thermal-equilibrium recombination rate for electrons and holes in this material.

<Solution>

(a) Recombination rates are equal

noτno=poτpo ,  no= Nd= 1016 cm-3

po=ni2no=(1.5 ×1010)21016=2.25 × 104 cm-3

Then 1016τno=2.25 ×10420 ×10-6 ,  τno=8.89 ×106 s

(b) Generation rate = recombination rate

Then  G=2.25 ×10420 ×10-6=1.125×109 cm-3s-1

(c) R=G= 1.125×109 cm-3s-1

Problem 4 (10 points)

A silicon sample at T = 300 K has a uniform acceptor concentration of 71015 cm-3. The excess carrier lifetime is τn0 = 10-7 s.

(a) Determine the ambipolar mobility.

(b) Find the ambipolar diffusion coefficient.

(c) What are the electron and hole lifetimes?

<Solution>

p-type material: minority carriers are electrons

a μ'= μn    from figure 5.3, μn ≅ 1300 cm2/Vs

b D'= Dn =kTeμn=0.02591300=33.67  cm2/s

c τnt= τno =10-7 s,      po= Na =7 ×1015 cm-3

no=ni2Na=(1.5 ×1010)27 ×1015=3.21 × 104 cm-3

noτnt=poτpt= 3.214 ×10410-7 = 7 ×1015τpt

τpt=2.18 ×104 s

Problem 5 (10 points)

Assume that an n-type semiconductor is uniformly illuminated, producing a uniform excess generation rate g'. Show that in steady state the change in the semiconductor conductivity is given by

∆ σ=e(μn+ μp)τp0g'

(Hint: In steady-state, δp = g'τp0)

<Solution>

σ=eμnn+ eμnp

With excess carriers

n=no+ δn and p=po+ δp

For an n-type semiconductor, we can write

δn= δp

Then

σ=eμnno+ δn+eμppo+ δp

or

σ=eμnno+eμppo+eμn+ μpδp

So

∆ σ=eμn+ μpδp

In steady-state,

δp = g'τp0

So that

∆ σ=e(μn+ μp)τp0g'

Problem 6 (10 points)

Consider a silicon sample at T = 300 K that is uniformly doped with acceptor impurity atoms at a concentration of Na = 1016 cm-3. At t = 0, a light source is turned on generating excess carriers uniformly throughout the sample at a rate of g’ = 81020 cm-3 s-1. Assume the minority carrier lifetime is τn0 = 510-7 s, and assume mobility values of μn = 900 cm2 /V-s and μp = 380 cm2 /V-s.

(a) Determine the conductivity of the silicon as a function of time for t0.

(b) What is the value of conductivity at (i) t = 0 and (ii) t = ꝏ?

<Solution>

Problem 7 (10 points)

In a GaAs material at T = 300 K, the doping concentrations are Nd = 81015 cm-3 and Na = 21015 cm-3. The thermal equilibrium recombination rate is Ro = 4104 cm-3s-1.

- What is the minority carrier lifetime?

- A uniform generation rate for excess carriers results in an excess carrier recombination rate of R'= 21021 cm-3 s-1. What is the steady-state excess carrier concentration?

- What is the excess carrier lifetime?

<Solution>

no= Nd- Na= 8×1015- 2×1015= 6×1015 cm-3

p0=ni2n0=1.8 × 10626 ×1015=5.4 × 10-4 cm-3

a Ro=p0τp0→ 4 × 104= 5.4 × 10-4τp0

so τp0=1.35 × 10-8 s

b δp= g'τp0=2 × 10211.35 × 10-8=2.7 × 1013 cm-3

c τ= τp0=1.35 × 10-8 s

Problem 8 (10 points)

Consider a bar of p-type silicon that is uniformly doped to a value of Na = 21016 cm-3 at T = 300 K. The applied electric field is zero. A light source is incident on the end of the semiconductor as shown in the figure below. The steady-state concentration of excess carriers generated at x = 0 0 is δp(0) = δn(0) = 21014 cm-3. Assume the following parameters: μn = 1200 cm2/V-s, μp = 400 cm2/V-s, τn0 = 10-6 s, and τp0 = 510-7 s. Neglecting surface effects,

- Determine the steady-state excess electron and hole concentrations as a function of distance into the semiconductor.

- Calculate the steady-state electron and hole diffusion current densities as a function of distance into the semiconductor.

<Solution>

Problem 9 (10 points)

Consider n-type GaAs doped at Nd = 1016 cm-3.

(a) For an excess concentration of p = (0.02)Nd, determine (i) EFn - EFi and (ii) EFi - EFp.

(b) Repeat part (a) if p = (0.1)Nd.

<Solution>

a i EFn-EFi= kTlnno+nni= 0.0259ln1.02 × 10161.8 × 106=0.58166 eV

ii EFi-EFp= kTlnδpni= 0.0259ln0.02 × 10161.8 × 106=0.47982 eV

b i EFn-EFi= kTlnno+nni= 0.0259ln1.1 × 10161.8 × 106=0.58361 eV

ii EFi-EFp= kTlnδpni= 0.0259ln0.1 × 10161.8 × 106=0.52151 eV

Problem 10 (10 points)

Consider p-type silicon at T = 300 K doped to Na = 51014 cm-3. Assume excess carriers are present and assume that EF - EFp = (0.01)kT.

(a) Does this condition correspond to low injection? Why or why not?

(b) Determine EFn - EFi.

<Solution>